Commander Package Install Guide

QUESTIONS?

Installation videos at

https://magnumtruckracks.com/installation

Thank you for purchasing the Magnum Commander Package

In the box:

(2) 6” Angled Light Brackets

(2) 2” x 5” Work Lights w/Brackets

(2) Corner Mount Amber Strobes w/ Magnetic Base

Installation:

- Mount the angle light brackets w/ t-bolt on each side of the rack

- The rack comes pre-wired to accept the accessories.

- On the front side of the rack light boxes there will be a white (ground) purple (work light) orange (strobe)

- The white (ground) will be used for both the strobes & work lights.

- Use supplied heat shrink connectors to connect the terminals.

- At the harness end there will be an orange (strobe) purple (work light) & red (6” oval strobe in rack) stubbed wire.

- Connect the wires to upfitter switches in cab. The orange & red wire for both sets of strobes can be combined to a single switch.